IJCSNS International Journal of Computer Science and Network Security, VOL.11 No.3, March 2011

177

Manuscript received March 5, 2011

Manuscript revised March 20, 2011

A Graphical User Interface for Modeling the Optical Properties of

Coated Metal Nanoparticles

H. A. Elmikaty

a

, A. S. Samra

b

, B. B. Yousif

c*

a,b

Electronics and Communications Dept., Faculty of Engineering- Mansoura University-Egypt

c

Electrical and Communications Dept.- Faculty of Engineering-. -Kafrelsheikh University- Egypt

Abstract

This paper presents a graphical user interface, based on modified

long-wavelength approximation me

thod (MLWA) to compute the

optical properties of core-shell nanoparticles. The validity of this

technique is verified by comparison to the exact solution (Mie

theory). For core-shell nanoparticles, when the shell thickness of a

core-shell particle is decreased, the plasmon resonance shifts to

longer wavelengths. This red shift is accompanied by an increase

in peak intensity. For coated spherical particles, we confirm that

our approach yields a correspondence with Mie theory and gives

an approximation error of less

than 2.8% for silica-gold and

silica-silver particles with outer diameters approaching 40nm and

26nm and inner diameters approaching 38nm and 24nm

respectively. Also the MLWA can give an acceptable degree of

accuracy 4.92% for larger part

icles of silica-gold with (inner-

outer) diameters 68-70nm at surface plasmon resonance

wavelength 1573nm. For this simulation run, the computation

time was less than 150.050 ms (Intel Pentium 4 CPU at 3GHz,

1.00GB of RAM), which includes calc

ulation of the polarizability

and all three cross-section plots of absorption, scattering and

extinction. This simulator provides a useful guide tool for optical

characterization experiments.

Keywords:

optical properties; coated nanoparticles; surface plasmo