RESEARCH PAPER

Optical responses of plasmonic gold nanoantennas through

numerical simulation

Bedir B. Yousif

•

Ahmed S. Samra

Received: 12 September 2012 / Accepted: 23 November 2012

Ó

Springer Science+Business Media Dordrecht 2012

Abstract

Nanoantennas for visible and infrared

radiation can strongly enhance the interaction of light

with nanoscale matter by their ability to efficiently link

propagating and spatially localized optical fields. Gold

nanoparticles are the material that is mostly used

experimentally, since it combines a favorable dielec-

tric function in the red and near-IR with excellent

chemical stability. So, the gold material is used to build

nanoantennas in this research. The optical properties of

plasmonic dimer nanoantennas are investigated in

detail using the finite integration technique. The

validity of this technique is verified by comparison to

the exact solution generalized Mie method (GMM).

The influence of the geometrical parameters (antenna

length, gap dimension, and shapes) on the antenna field

enhancement and spectral response is discussed.

Localized surface plasmon resonances of Au (gold)

dimers nanospheres, bowtie and aperture bowtie

nanoantennas are modeled. The enhanced field is

equivalent to a strong light spot which can lead to the

resolution improvement of the microscopy and optical

lithography, thus increasing the optical data storage

capacity. Furthermore, the sensitivity of the antennas

to index changes of the environment and substrate is

investigated in detail for biosensing applications. We

confirm that our approach yields an exact correspon-

dence with GMM theory, for Au dimers nanospheres at

gap dimension 5 and 10 nm but gives an approximation

error of less than 1.37 % for gap dimension 1 and 2 nm

with diameters approaching 80 nm. In addition, the

far-field characteristics of the aperture bowtie nano-

antenna such as directivity and gain are studied. The

promising results of this study may have useful

potential applications in near-field sample detection,

optical microscopy, etc.

Keywords

Optical antennas

Near-field optics

Surface plasmon resonance